Testing Aluminum Silicon Carbide to Explore Alternate Electrode Materials in the Electrical Discharge Manufacturing Process

Nick Thompson

P.V. Chaitanya Kumar

Aum Patel

April 2023

Introduction

Electrical Discharge Manufacturing (EDM) is a non-conventional machining process where electricity is applied over a small period to a material submerged in a dielectric solution.  The arc produced between the electrode and the material removes a surface layer of the material.   This process is primarily used to precisely manufacture hard materials such as tungsten or steel, where traditional machining processes such as milling or grinding would be inefficient.  EDM is used in the aerospace industry to reduce the surface roughness at a microscopic level, and therefore friction, of hard components found in airplane engines.  Different materials are used for electrodes, with different benefits.  Two common materials are copper and brass.  Copper is inexpensive and resists erosion but difficult to machine.  Brass is much easier to machine but wears out faster than Copper and is more expensive.  Researchers are exploring Aluminum Silicon Carbide composites as an alternative to Brass and Copper as it is easier to machine than copper and less expensive than brass.

Experimental Design

Researchers conducted EDM experiments on pieces of 080M40 Steel submerged in dielectric oil with a specific gravity of 0.763 using four different factors: Tool material, Pulse on Time, Current, and Pulse off time (Figure 2).  The primary factor of interest is the electrode tool material.  The electrode tool materials were Copper, Brass, Aluminum + 3% Silicon Carbide, Aluminum + 6% Silicon Carbide and Aluminum + 9% Silicon Carbide (Figure 1).  The Pulse on Time factor levels were 200, 500 and 900 microseconds. The Current factors used were 6, 8 and 10 amps.  The Pulse Off Time factor levels were 100, 200 and 500 microseconds.  The response the researchers measured was the surface roughness(SR) of the steel following the experiment, measured in micrometers (Figure 3).   The researchers conducted 45 experiments of individual replicates based on the principles of the Taguchi design, with each tool material level containing L9 experiments(each factor level appears 3 times).   This allowed the researchers to conduct 45 experiments instead of the 135 required by a full factorial design with 1 replicate.   Our team analyzed the surface roughness results using ANOVA in Minitab.  Each factor had discrete, rather than continuous levels so multiple linear regression was not appropriate.  We decided to use forward selection (α = 0.15) and then Tukey tests to determine which factors and interactions resulted in the lowest surface roughness.  For this experiment, a lower mean surface roughness (smooth finish) is considered better.

| Factor Level | Tool Material | Pulse on Time(μs) | Current(A) | Pulse off Time(μs) |
| --- | --- | --- | --- | --- |
| 1 | Al + 3%SiC | 200 | 6 | 100 |
| 2 | Al + 6%SiC | 500 | 8 | 200 |
| 3 | Al + 9%SiC | 900 | 10 | 500 |
| 4 | Copper | - | - | - |
| 5 | Brass | - | - | - |

Results

See Appendix 1 for data results.

WORKSHEET 1

General Linear Model: SR versus Tool, Pulse on time, Current, Pulse off time

Method

| Factor coding | (-1, 0, +1) |
| --- | --- |

Forward Selection of Terms

α to enter = 0.15

Factor Information

| Factor | Type | Levels | Values |
| --- | --- | --- | --- |
| Tool | Fixed | 5 | Al6061 + 3 % SiC, Al6061 + 6% SiC, Al6061 + 9% SiC, Brass, Copper |
| Pulse on time | Fixed | 3 | 200, 500, 900 |
| Current | Fixed | 3 | 6, 8, 10 |

Analysis of Variance

| Source | DF | Adj SS | Adj MS | F-Value | P-Value |
| --- | --- | --- | --- | --- | --- |
| Tool | 4 | 126.431 | 31.6077 | 51.23 | 0.000 |
| Pulse on time | 2 | 36.727 | 18.3634 | 29.76 | 0.000 |
| Current | 2 | 12.428 | 6.2142 | 10.07 | 0.001 |
| Tool*Pulse on time | 8 | 11.959 | 1.4949 | 2.42 | 0.063 |
| Tool*Current | 8 | 9.757 | 1.2196 | 1.98 | 0.117 |
| Pulse on time*Current | 4 | 7.419 | 1.8547 | 3.01 | 0.050 |
| Error | 16 | 9.871 | 0.6170 |  |  |
| Total | 44 | 214.592 |  |  |  |

From these results, we see that Pulse Off Time did not have a significant effect on surface roughness at all.  From the remaining factors, Tool Material, Pulse On Time, and Current were very significant at the 95% confidence level with p-values well below α = 0.05 and close to 0.  The interaction between Pulse On Time and Current is almost significant with a p-value of 0.050, but we excluded it because p is not less than 0.05.

Model Summary

| S | R-sq | R-sq(adj) | R-sq(pred) |
| --- | --- | --- | --- |
| 0.785473 | 95.40% | 87.35% | 63.61% |

The results indicate that 87.35% of the surface roughness of the steel can be attributed to the factors.

Coefficients

| Term | Coef | SE Coef | T-Value | P-Value | VIF |
| --- | --- | --- | --- | --- | --- |
| Constant | 6.599 | 0.117 | 56.35 | 0.000 |  |
| Tool |  |  |  |  |  |
| Al6061 + 3 % SiC | 2.034 | 0.234 | 8.69 | 0.000 | 1.60 |
| Al6061 + 6% SiC | -2.247 | 0.234 | -9.59 | 0.000 | 1.60 |
| Al6061 + 9% SiC | 0.640 | 0.234 | 2.73 | 0.015 | 1.60 |
| Brass | -1.690 | 0.234 | -7.22 | 0.000 | 1.60 |
| Pulse on time |  |  |  |  |  |
| 200 | -1.190 | 0.166 | -7.18 | 0.000 | 1.33 |
| 500 | 0.191 | 0.166 | 1.16 | 0.265 | 1.33 |
| Current |  |  |  |  |  |
| 6 | -0.423 | 0.166 | -2.56 | 0.021 | 1.33 |
| 8 | -0.317 | 0.166 | -1.92 | 0.073 | 1.33 |
| Tool*Pulse on time |  |  |  |  |  |
| Al6061 + 3 % SiC 200 | -0.389 | 0.331 | -1.17 | 0.258 | 2.13 |
| Al6061 + 3 % SiC 500 | -0.282 | 0.331 | -0.85 | 0.408 | 2.13 |
| Al6061 + 6% SiC 200 | 0.200 | 0.331 | 0.60 | 0.554 | 2.13 |
| Al6061 + 6% SiC 500 | -0.773 | 0.331 | -2.34 | 0.033 | 2.13 |
| Al6061 + 9% SiC 200 | -0.277 | 0.331 | -0.84 | 0.416 | 2.13 |
| Al6061 + 9% SiC 500 | 0.543 | 0.331 | 1.64 | 0.121 | 2.13 |
| Brass 200 | -0.022 | 0.331 | -0.07 | 0.947 | 2.13 |
| Brass 500 | -0.113 | 0.331 | -0.34 | 0.737 | 2.13 |
| Tool*Current |  |  |  |  |  |
| Al6061 + 3 % SiC 6 | -0.212 | 0.331 | -0.64 | 0.531 | 2.13 |
| Al6061 + 3 % SiC 8 | 0.774 | 0.331 | 2.34 | 0.033 | 2.13 |
| Al6061 + 6% SiC 6 | 0.100 | 0.331 | 0.30 | 0.766 | 2.13 |
| Al6061 + 6% SiC 8 | -0.023 | 0.331 | -0.07 | 0.945 | 2.13 |
| Al6061 + 9% SiC 6 | -0.118 | 0.331 | -0.36 | 0.727 | 2.13 |
| Al6061 + 9% SiC 8 | 0.202 | 0.331 | 0.61 | 0.551 | 2.13 |
| Brass 6 | 0.803 | 0.331 | 2.42 | 0.028 | 2.13 |
| Brass 8 | -0.705 | 0.331 | -2.13 | 0.049 | 2.13 |
| Pulse on time*Current |  |  |  |  |  |
| 200 6 | -0.005 | 0.234 | -0.02 | 0.985 | 1.78 |
| 200 8 | 0.524 | 0.234 | 2.24 | 0.040 | 1.78 |
| 500 6 | 0.405 | 0.234 | 1.73 | 0.103 | 1.78 |
| 500 8 | -0.251 | 0.234 | -1.07 | 0.299 | 1.78 |

The VIF for each term in the regression equation is less than 4, so we do not need to conduct additional investigations.

Regression Equation

| SR | = | 6.599 + 2.034 Tool_Al6061 + 3 % SiC - 2.247 Tool_Al6061 + 6% SiC + 0.640 Tool_Al6061 + 9% SiC - 1.690 Tool_Brass + 1.263 Tool_Copper - 1.190 Pulse on time_200 + 0.191 Pulse on time_500 + 0.998 Pulse on time_900 - 0.423 Current_6 - 0.317 Current_8 + 0.741 Current_10 - 0.389 Tool*Pulse on time_Al6061 + 3 % SiC 200 - 0.282 Tool*Pulse on time_Al6061 + 3 % SiC 500 + 0.670 Tool*Pulse on time_Al6061 + 3 % SiC 900 + 0.200 Tool*Pulse on time_Al6061 + 6% SiC 200 - 0.773 Tool*Pulse on time_Al6061 + 6% SiC 500 + 0.573 Tool*Pulse on time_Al6061 + 6% SiC 900 - 0.277 Tool*Pulse on time_Al6061 + 9% SiC 200 + 0.543 Tool*Pulse on time_Al6061 + 9% SiC 500 - 0.266 Tool*Pulse on time_Al6061 + 9% SiC 900 - 0.022 Tool*Pulse on time_Brass 200 - 0.113 Tool*Pulse on time_Brass 500 + 0.135 Tool*Pulse on time_Brass 900 + 0.487 Tool*Pulse on time_Copper 200 + 0.625 Tool*Pulse on time_Copper 500 - 1.113 Tool*Pulse on time_Copper 900 - 0.212 Tool*Current_Al6061 + 3 % SiC 6 + 0.774 Tool*Current_Al6061 + 3 % SiC 8 - 0.561 Tool*Current_Al6061 + 3 % SiC 10 + 0.100 Tool*Current_Al6061 + 6% SiC 6 - 0.023 Tool*Current_Al6061 + 6% SiC 8 - 0.077 Tool*Current_Al6061 + 6% SiC 10 - 0.118 Tool*Current_Al6061 + 9% SiC 6 + 0.202 Tool*Current_Al6061 + 9% SiC 8 - 0.084 Tool*Current_Al6061 + 9% SiC 10 + 0.803 Tool*Current_Brass 6 - 0.705 Tool*Current_Brass 8 - 0.098 Tool*Current_Brass 10 - 0.573 Tool*Current_Copper 6 - 0.248 Tool*Current_Copper 8 + 0.821 Tool*Current_Copper 10 - 0.005 Pulse on time*Current_200 6 + 0.524 Pulse on time*Current_200 8 - 0.519 Pulse on time*Current_200 10 + 0.405 Pulse on time*Current_500 6 - 0.251 Pulse on time*Current_500 8 - 0.154 Pulse on time*Current_500 10 - 0.401 Pulse on time*Current_900 6 - 0.273 Pulse on time*Current_900 8 + 0.673 Pulse on time*Current_900 10 |
| --- | --- | --- |

Fits and Diagnostics for Unusual Observations

| Obs | SR | Fit | Resid | Std Resid |  |
| --- | --- | --- | --- | --- | --- |
| 32 | 6.854 | 7.862 | -1.008 | -2.15 | R |
| 44 | 3.700 | 4.747 | -1.047 | -2.24 | R |

R  Large residual

Here we see the 32nd and 44th observation to have a standardized residual less than -2, which means that these data points can be considered outliers.

The 4-in-1 residual plot comparison shows that ANOVA is a good method to use in the analysis.  The Normal probability plot shows the relationship between the residuals follows a linear pattern.  The Residual histogram shows the residuals are normally distributed.  The Versus Fits plot has an even spread of points above and below zero, showing the residuals are random.  The Versus Order plot has no discernible pattern, showing the residuals are independent of each other.

WORKSHEET 1

Comparisons for SR

Tukey Pairwise Comparisons: Tool

Grouping Information Using the Tukey Method and 95% Confidence

| Tool | N | Mean | Grouping |  |  |
| --- | --- | --- | --- | --- | --- |
| Al6061 + 3 % SiC | 9 | 8.63267 | A |  |  |
| Copper | 9 | 7.86156 | A | B |  |
| Al6061 + 9% SiC | 9 | 7.23867 |  | B |  |
| Brass | 9 | 4.90833 |  |  | C |
| Al6061 + 6% SiC | 9 | 4.35167 |  |  | C |

Means that do not share a letter are significantly different.

Tukey Pairwise Comparisons: Pulse on time

Grouping Information Using the Tukey Method and 95% Confidence

| Pulse on time | N | Mean | Grouping |  |  |
| --- | --- | --- | --- | --- | --- |
| 900 | 15 | 7.59687 | A |  |  |
| 500 | 15 | 6.78993 |  | B |  |
| 200 | 15 | 5.40893 |  |  | C |

Means that do not share a letter are significantly different.

Tukey Pairwise Comparisons: Current

Grouping Information Using the Tukey Method and 95% Confidence

| Current | N | Mean | Grouping |  |
| --- | --- | --- | --- | --- |
| 10 | 15 | 7.33927 | A |  |
| 8 | 15 | 6.28127 |  | B |
| 6 | 15 | 6.17520 |  | B |

Means that do not share a letter are significantly different.

Based on the Tukey Tests conducted at 95% confidence on the significant factors, we find that Brass and Aluminum with 6% Silicon Carbide resulted in the lowest mean surface roughness at 4.90833 and 4.35167 micrometers, respectively.  A Pulse On Time of 200 microseconds resulted in the lowest surface roughness with a mean of 5.40893 micrometers.  For current, 6 and 8 amps both resulted in the lowest surface roughness with 6.28127 and 6.17520 micrometers, respectively.

These results are also reflected in the Main Effects Plot which show that 200 microsecond Pulse On Time, 6 and 8 amps Current and Brass and Aluminum with 6% Silicon Carbide Tool Material result in the lowest mean surface roughness.

WORKSHEET 1

Prediction for SR

General Linear Model Information

Terms

Tool Pulse on time Current

Settings

| Variable | Setting |
| --- | --- |
| Tool | Al6061 + 6% SiC |
| Pulse on time | 200 |
| Current | 6 |

Prediction

| Fit | SE Fit | 95% CI | 95% PI |
| --- | --- | --- | --- |
| 2.73864 | 0.465510 | (1.79455, 3.68274) | (0.426085, 5.05120) |

Settings

| Variable | Setting |
| --- | --- |
| Tool | Al6061 + 6% SiC |
| Pulse on time | 200 |
| Current | 8 |

Prediction

| Fit | SE Fit | 95% CI | 95% PI |
| --- | --- | --- | --- |
| 2.84471 | 0.465510 | (1.90061, 3.78881) | (0.532151, 5.15727) |

Settings

| Variable | Setting |
| --- | --- |
| Tool | Brass |
| Pulse on time | 200 |
| Current | 6 |

Prediction

| Fit | SE Fit | 95% CI | 95% PI |
| --- | --- | --- | --- |
| 3.29531 | 0.465510 | (2.35121, 4.23941) | (0.982751, 5.60787) |

Settings

| Variable | Setting |
| --- | --- |
| Tool | Brass |
| Pulse on time | 200 |
| Current | 8 |

Prediction

| Fit | SE Fit | 95% CI | 95% PI |
| --- | --- | --- | --- |
| 3.40138 | 0.465510 | (2.45728, 4.34548) | (1.08882, 5.71394) |

Conducting a response prediction in Minitab based on the significant factor levels, we found the following.

Mean SR (Al + 6% SiC, 200μs, 6A) = 2.73864 μm, 95% CI (1.79455, 3.68274)

Mean SR (Al + 6% SiC, 200μs, 8A) = 2.84471 μm, 95% CI (1.90061, 3.78881)

Mean SR (Brass, 200μs, 6A) = 3.29531 μm, 95% CI (2.35121, 4.23941)

Mean SR (Brass, 200μs, 8A) = 3.40138 μm, 95% CI (2.45728, 4.34548)

Conclusion

We conclude that electrodes made of Aluminum + 6% Silicon Carbide and Brass result in the lowest surface roughness.  We also found that currents of 6 and 8 Amps, and a Pulse on Time of 200 μs resulted in the lowest surface roughness.  We believe that constructing electrodes made from a composite material of Aluminum + 6% Silicon Carbide is a suitable replacement for Brass in the Electrical Discharge Manufacturing process.

References

1.  B Kishan,  B Sudheer Prem Kumar , S Gajanana, B Ravi Kumar and P V K Chaitanya Kumar, Regression Model for Surface Roughness on EDM using Al6061 + SiC, Copper and Brass Tools, Manufacturing Technology Today, Vol. 18, No. 9, September 2019.

2.  Asif Rashid, Muhammad P. Jahan, Chapter 2 - Microfabrication by Electrical Discharge Machining-Based Hybrid Processes, Pg 33-62, In Micro and Nano Technologies, Micro Electro-Fabrication, Elsevier, 2021.

3.  EDM Electrode Materials Information, https://www.globalspec.com/learnmore/

materials_chemicals_adhesives/electrical_optical_specialty_materials/electrical_contact_electrode_materials/edm_electrode_materials, April 2023.

4.  Taguchi designs, https://support.minitab.com/en-us/minitab/21/help-and-how-to/statistical-modeling/doe/supporting-topics/taguchi-designs/taguchi-designs, April 2023.

Appendix 1: Run order and Response Data

| Tool | Pulse on time | Current | Pulse off time | SR |
| --- | --- | --- | --- | --- |
| Al6061 + 3 % SiC | 200 | 6 | 100 | 6.947 |
| Al6061 + 3 % SiC | 200 | 8 | 200 | 7.805 |
| Al6061 + 3 % SiC | 200 | 10 | 500 | 6.411 |
| Al6061 + 3 % SiC | 500 | 6 | 500 | 8.69 |
| Al6061 + 3 % SiC | 500 | 8 | 100 | 8.512 |
| Al6061 + 3 % SiC | 500 | 10 | 200 | 8.425 |
| Al6061 + 3 % SiC | 900 | 6 | 200 | 8.354 |
| Al6061 + 3 % SiC | 900 | 8 | 500 | 10.95 |
| Al6061 + 3 % SiC | 900 | 10 | 100 | 11.6 |
| Al6061 + 6% SiC | 200 | 6 | 100 | 2.859 |
| Al6061 + 6% SiC | 200 | 8 | 200 | 3.203 |
| Al6061 + 6% SiC | 200 | 10 | 500 | 4.025 |
| Al6061 + 6% SiC | 500 | 6 | 500 | 3.675 |
| Al6061 + 6% SiC | 500 | 8 | 100 | 3.234 |
| Al6061 + 6% SiC | 500 | 10 | 200 | 4.4 |
| Al6061 + 6% SiC | 900 | 6 | 200 | 5.551 |
| Al6061 + 6% SiC | 900 | 8 | 500 | 5.597 |
| Al6061 + 6% SiC | 900 | 10 | 100 | 6.621 |
| Al6061 + 9% SiC | 200 | 6 | 100 | 5.24 |
| Al6061 + 9% SiC | 200 | 8 | 200 | 6.038 |
| Al6061 + 9% SiC | 200 | 10 | 500 | 6.039 |
| Al6061 + 9% SiC | 500 | 6 | 500 | 7.303 |
| Al6061 + 9% SiC | 500 | 8 | 100 | 7.874 |
| Al6061 + 9% SiC | 500 | 10 | 200 | 8.742 |
| Al6061 + 9% SiC | 900 | 6 | 200 | 7.55 |
| Al6061 + 9% SiC | 900 | 8 | 500 | 7.458 |
| Al6061 + 9% SiC | 900 | 10 | 100 | 8.904 |
| Copper | 200 | 6 | 100 | 6.05 |
| Copper | 200 | 8 | 200 | 7.705 |
| Copper | 200 | 10 | 500 | 7.723 |
| Copper | 500 | 6 | 500 | 8.426 |
| Copper | 500 | 8 | 100 | 6.854 |
| Copper | 500 | 10 | 200 | 10.754 |
| Copper | 900 | 6 | 200 | 6.119 |
| Copper | 900 | 8 | 500 | 7.33 |
| Copper | 900 | 10 | 100 | 9.793 |
| Brass | 200 | 6 | 100 | 3.809 |
| Brass | 200 | 8 | 200 | 3.326 |
| Brass | 200 | 10 | 500 | 3.954 |
| Brass | 500 | 6 | 500 | 5.766 |
| Brass | 500 | 8 | 100 | 4.633 |
| Brass | 500 | 10 | 200 | 4.561 |
| Brass | 900 | 6 | 200 | 6.289 |
| Brass | 900 | 8 | 500 | 3.7 |
| Brass | 900 | 10 | 100 | 8.137 |